# CYNGOR CYMUNED LLANISMEL

ST ISHMAEL COMMUNITY COUNCIL

Cadeirydd / Chair:

Cllr Alicia Dalton

Julie Rees

Hafan Dawel

Mynydd Cerrig

Llanelli

Carmarthenshire

SA15 5BD

Tel. 07825 003648

Ebost / e-mail: clerk@stishmaelscc.org.uk

www.stishmaelscc.org.uk

PUBLIC NOTICE

## TENDER 3 – FISHERMAN CORNER RENOVATION

TENDERS are invited for the RECONSTRUCTION and subsequent CLEARING and REMOVAL (all waste to be disposed of appropriately) of the surrounding quadrant of the Fisherman Corner opposite the White Lion, Ferryside by 31st March 2023.

- Rake out, re-point and make good the slabs laid (as indicated on the plan as Area A).

- Dig out and dispose of current tiles and concrete to solid ground to a depth of at least 100mm (as indicated on the plan as Area B).

- Compact the base of the ground in Area B with a plate compactor.

- Lay suitable terram type membrane over Area B.

- Lay (A142) light gauge steel mesh over Area B to the levels required of spacers.

- Set 4-inch sewer pipes around the whole perimeter (Area A and B) to accommodate 13 timer posts. Pipes should be approximately 150mm in length.

- Thirteen posts of tanalised timber with a hole for the rope will be required (some in situ may be reusable). The hole for rope will need to be approx. 30mm.

- Lay concrete (strong 1,2,4) to a minimum depth of 100mm on the whole of Area B. Concrete should be to a reasonable finish.

- Beach type stones to pushed in and set in the concrete to cover all of Area B. Some of the stones in situ may be reusable.

- Retain golden grassed only, remaining plants /weeds to be removed and disposed of appropriately.

- Please note the rope will be supplied and positioned later.

- It should be noted that all works specified should be carried out while maintaining the integrity and fabric of the existing wooden Fisherman Sculpture.

- The site and surrounding area should be left clean and tidy.

- All relevant health and safety guidance should be followed and risk assessments carried out.

The Tenders must be returned to the undersigned no later than 5.00 p.m. 31st November 2022, in a sealed envelope clearly marked “TENDER”.  Any envelope not so endorsed will not be considered.

The Council does not bind itself to accept the lowest or any tender and payment will not be made until works have been completed, inspected, and approved.

Tender price quoted should be net of VAT and the VAT status of the person submitting the Tender must be made known to the Council with the VAT number provided where appropriate.

Tenderers are invited to propose a stage payment plan if required.

A copy of a current Public Liability Policy must be submitted with the Tender

A copy of a current Waste Carriers Licence must be submitted with the Tender.

PAYMENT MAY BE AFFECTED IF THE FOLLOWING CONDITIONS ARE NOT MET:

- Contractor must notify the Clerk once the work is completed.

- Any substandard works notified to the contractor following inspection must be rectified at the earliest opportunity.

- Should the Community Council be dissatisfied with the standard of work, the Community Council would be able to cancel the contract at any time.

Julie Rees

Clerk

Plan of Fisherman Corner Renovation

5.40m

2.90m

Area A	1.7m

6.40m

Area B

Curve = 9.70m